SERVICE UPDATE

FROM

NEW YORK BUS SALES, LLC

***********************

Complaint: 	A3FE and A3RE units experience erratic headlight (+/or) taillight operation or the headlight circuit fails causing no head/tail lights.

Cause:	There is a diode located in the power wire feeding the headlight circuit and it can become hot causing the erratic behavior or just burn out.

Correction: 	At the headlight switch back 4”-6” in the wire harness in the power wire you will find the diode.

Mount the rectifier (#35MB100) shown On the attachment to a steel surface and extend the wires to the rectifier from the switch and to the headlights as shown.

This Complaint/Correction was provided by OUR OWN Brian Lamaitis

THANK YOU BRIAN!!

If you find things that might help others in the industry please call or e-mail us:

CHITTENANGO – phone the Service Department at 800-962-5768 or

Daryl Wallace @ dwallace@newyorkbussales.com

Brian Lamaitis @ blamaitis@newyorkbussales.com

ROCHESTER – phone the Service Department at 800-463-3232 or

Dan Quibell @ dpquibell@newyorkbussales.com

Gary Chichester @ gchichester@newyorkbussales.com

ALBANY – phone the Service Department at 866-867-1100 or

Steve Searles @ ssearles@newyorkbussales.com