SERVICE UPDATE

FROM

NEW YORK BUS SALES, LLC

***********************

Complaint: 	Units experience what appears to be an alternator failure (either high or low voltage).Alternators are Leece-Neville 175amp.  Most issues have been with the A3FE units.

Cause:	The internal regulator on the alternator is failing from an overheat situation.

Correction: 	Blue Bird offers the attached “retro – fit” kit which replaces the internal regulator with an external regulator and they are asking to have it mounted on the first chassis outrigger below the alternator – this will assure sufficient cooling.

These Parts Will Be Provided Under Warranty

THIS WILL BE A STANDARD PRODUCTION FEATURE ON A3FE STARTING IN MAY 2005

If you find things that might help others in the industry please call or e-mail us:

CHITTENANGO – phone the Service Department at 800-962-5768 or

Daryl Wallace @ dwallace@newyorkbussales.com

Brian Lamaitis @ blamaitis@newyorkbussales.com

ROCHESTER – phone the Service Department at 800-463-3232 or

Dan Quibell @ dpquibell@newyorkbussales.com

ALBANY – phone the Service Department at 866-867-1100 or

Steve Searles @ ssearles@newyorkbussales.com