SERVICE UPDATE

FROM

NEW YORK BUS SALES, LLC

***********************

Complaint: 	Belt squeal - A/C Compressor failure prematurely on GM  Microbirds

Cause:	When the front dash A/C unit is turned on separately from the rear, pressure in the system builds causing the compressor to overwork and cause failure

Correction: 	When these units were originally installed there was a tag placed on the dash stating that “both” systems needed to be “turned on” at the same time. With the loss of those tags we have seen failures due to operation error and have come up with a “re-wire” installing a 6 AMP diode that turns on the rear condensers thus relieving pressure in the system and alleviating the possibility of compressor failure

If you find things that might help others in the industry please call or e-mail us:

CHITTENANGO – phone the Service Department at 800-962-5768 or

Daryl Wallace @ dwallace@newyorkbussales.com

Brian Lamaitis @ blamaitis@newyorkbussales.com

ROCHESTER – phone the Service Department at 800-463-3232 or

Dan Quibell @ dpquibell@newyorkbussales.com

Gary Chichester @ gchichester@newyorkbussales.com

ALBANY – phone the Service Department at 866-867-1100 or

Steve Searles @ ssearles@newyorkbussales.com