SERVICE UPDATE

FROM

NEW YORK BUS SALES, LLC

Product Effected – Possibly Any Unit Delivered

Complaint: 	When using the warning light “cancel” switch is pressed rapidly 2 or more times ALL warning lights faintly flash and the stop arms “wave” and light up

Cause:	With the way that we initially wired the warning light cancel switch there is residual power that runs the system until it is dispersed

Correction: 	Remove the wires from where the warning light switch was initially connected. Go to Relay #P101 and find wire #1350B which is Red with a White tracer and wire so that the warning light cancel switch is in that circuit.

This Service Update was provided to you by both New York Bus Sales, LLC shops and the help of Rene Dykes from Blue Bird

If you find things that might help others in the industry please call or e-mail us:

CHITTENANGO – phone the Service Department at 800-962-5768 or

Daryl Wallace :  dwallace@newyorkbussales.com

Brian Lamaitis :  blamaitis@newyorkbussales.com

ROCHESTER – phone the Service Department at 800-463-3232 or

Dan Quibell :  dpquibell@cnycoach.com

Steve Barnhart :  smbarnhart@cnycoach.com