SERVICE NOTICE

IT HAS BEEN BROUGHT TO OUR ATTENTION THAT THERE HAVE BEEN SOME ISSUES SURROUNDING THE TURBO EXHAUST PIPE HANGER CRACKING ON THE A3FE PRODUCT

THIS HANGER MOUNTS TO THE CATERPILLAR BELLHOUSING AND A 4” U-BOLT CLAMP HOLDS THE TURBO PIPE TO THE HANGER. THE HANGER CRACKS AT THE 90 DEGREE BEND CLOSEST TO THE ENGINE.

IF YOU HAVE UNITS EXPERIENCING THIS ISSUE WE ARE ASKING THAT YOU EITHER WELD THE BRACKET AND ADD A “GUSSET” OR TO THAT YOU REPLACE THE BRACKET WITH ONE WHICH WE MANUFACTURE (#NYB003) AND THAT YOU CLOSELY LOOK AT THE FLANGE OF THE TURBO PIPE TO ASSURE IT HAS NOT CRACKED OR SEPARATED

BLUE BIRD WILL BE UPDATING THE BRACKET AND WE WILL MAKE YOU AWARE WHEN THE CHANGE TAKES PLACE

REVISION – DATE 11/9/05 – IT WAS BROUGHT TO OUR ATTENTION BY VIC TRIPP AT CARTHAGE C.S. THAT ON SOME UNITS WHEN THE CRACKED BRACKET IS REPLACED IT MAY BE NECESSARY TO SHIM THE EXHAUST U-CLAMP TO MEET THE BRACKET OR YOU WILL BE CREATING A BIND, WHICH WHEN COMBINED WITH ENGINE VIBRATION COULD CREATE PREMATURE FAILURE – THANKS VIC!!

THE FOLLOWING PARTS “MAY” BE REQUIRED

0002385(NYBS003)		BRKT – TURBO SUPPORT

1005552				4” BAND CLAMP

0000707				PIPE ASSY-TURBO

1250646				4” TURBO EXHAUST CLAMP

1444009				EXHAUST WRAP-50’ROLL

1124767				FLEX CONNECTOR

CHITTENANGO      800-962-5768

HENRIETTA         800-463-3232

ALBANY             866-867-1100

THANK YOU

NEW YORK BUS SALES, LLC.