Tech Tip

From your friends at New York Bus Sales

Product Affected-Units with New Emissions and Diesel Engines

Complaint - 	The exhaust manifold seems to be leaking as there is a black “soot” trail forming but there does not seem to be a leak-

Cause -	With the new emissions not producing “soot” as in the past, when the engines are cold the metals “shrink” forming slight gaps where when started the exhaust leaks out and causes these streaks.

Correction - 	Make sure that you are checking for exhaust leaks with the engine at operating temperature – This way you know that the metals have expanded to their full size. Even with the slightest contraction and expansion when sealing these the manifolds may again leak when cold and then seal when at operating temperature.

Contact Our Service Department With Any Questions

CHITTENANGO – phone 800-962-5768 or

Daryl Wallace :  dwallace@newyorkbussales.com

Brian Lamaitis :  blamaitis@newyorkbussales.com

ROCHESTER – phone 800-463-3232 or

Gary Chichester: gchichester@newyorkbussales.com

ALBANY – phone 866-867-1100 or

Ben Reiling: breiling@newyorkbussales.com

Steve Searles: ssearles@newyorkbussales.com

WARRANTY – 800-962-5768 ext 237

Morgan Jenkins: mjenkins@newyorkbussales.com

THANKS TO RICH DENNISH FROM

AMERICAN BUS AND COACH FOR POINTING THIS OUT TO US!!!

ALL of our Tech Tips can be found on the New York Bus Sales website at

http://www.newyorkbussales.com/pages/bulletins.cfm

Or at the New York Head Mechanic website at

http://www.nyhma.org/viewforum.php?f=2&start=0